Calculate the density of a laboratory sample

Density = Mass/ volume

You will use Data Studio and a force sensor to do this experiment.

Procedure:

- Attach the force sensor to the USB port and plug that into the USB socket on the computer.

- Start Data Studio.  Drag the force sensor icon down to the Digits icon in order to get a digital indication of the reading of the force sensor.

- Attach the force sensor to the sample using a loop of string.

- Determine and record the DRY weight of the sample = __________  Newtons  [A]

- Convert [A] to kilograms by dividing it by 9.8 = ______________kg [B]

- Convert [B] to grams by multiplying by 1000 = ______________g [C]

- Suspend the sample in water so that it is totally immersed, but NOT touching the bottom.

- Determine and record the Submerged weight of the sample = _________________ Newtons. [D]

- Convert [D] into kilograms by dividing it by 9.8 = ________________kg [E]

- Convert [E] into grams by multiplying by 1000 = ________________g [F]

- The buoyant force is equal to the apparent loss of weight = [C] – [F] = ________g  [G]

- The buoyant force in grams [G]= volume in cm3  = __________  cm3  [H]

- Determine the density of the sample = [C]/[H]  = _______________ g/ cm3

Report your results by filling out the form located at

https://spreadsheets.google.com/viewform?formkey=dDF5d1VpNEdQanlDNjZmcE5ONnZsdEE6MQ&ifq