# Weight VS Mass  (Capstone)

# Experimental Setup Capstone:

- Connect the Force Sensor to the USBLINK and plug that into the USB port of the computer.

- Use a c-clamp to attach a gallows pole socket to the desk and insert a gallows pole.

- Suspend the force sensor so that the hook is pointing down.

- Attach a small length (5-6 cm) of string to the hook on the Force Sensor.

- Start CAPSTONE. Choose “Two large digits”

- Click on “Select Measurement” then “Force”

You are now ready to begin collecting data.

Data Collection Procedure:

- Push the zero (tare) button on the force sensor.

- Attach a 50 gram hanger to the string on the Force Sensor and then add a 50-gram mass on the mass hanger. Record the weight shown on the digit display in the data table.

- Also record the amount of mass hanging from the Force Sensor in the data table. Don't forget to include the mass of the hanger itself.  Record the mass in kilograms.

- Repeat steps 2-3 with increasing amounts of hanging mass for a total of at least 8 data pairs.

- Enter your data into an  Xcel spreadsheet.

- Create a scatter chart in Xcel and add a Trendline (linear) and have the equation and R 2 .printed on the graph.

- Add the appropriate axis labels and chart label.

DATA TABLE

| Mass (kilograms) | Force (Newtons) |
| --- | --- |

Submit your results using the following link:  https://docs.google.com/forms/d/1yJgG8nXkvQKg0MmA6BXCahW2f2bXmNKMrgn73Jshtsk/viewform?usp=send_form

Conclusions/Questions:

- Write a sentence to describe the meaning of the weight of an object. Keep in mind that the weight was measured by the force sensor.

- In general, what pattern do you notice between weight of an object and its mass?

- What is the physical meaning of the slope for the weight-mass graph? (Hint: Look at the units!)